| Architecture | No. of Bits | ROM | RAM | OS Support |
| --- | --- | --- | --- | --- |
| 8051 | 8-bit | 4 KB (typical on-chip) | 128 B | ❌ No |
| PIC (PIC16 series) | 8-bit | 1 KB – 32 KB Flash | 64 B – 2 KB | ❌ No |
| AVR (ATmega328P) | 8-bit | 32 KB Flash | 2 KB | ❌ No |
| ARM7 LPC2148 | 32-bit | 512 KB Flash | 40 KB SRAM | ⚠️ Limited RTOS support |
| ARM Cortex-M4 | 32-bit | 64 KB – 2 MB Flash (depends on MCU) | 16 KB – 512 KB RAM (depends on MCU) | ✅ RTOS Support |

Quick Comparison

| Feature | 8051 | PIC | AVR | LPC2148 | Cortex-M4 |
| --- | --- | --- | --- | --- | --- |
| Architecture | Harvard | Harvard | Modified Harvard | ARM7TDMI | ARM Cortex-M4 |
| Clock Speed | Up to 33 MHz | Up to 64 MHz | Up to 20 MHz | Up to 60 MHz | Up to 200+ MHz |
| Multitasking OS | No | No | No | Limited | Yes |
| RTOS Support | No | Limited | Limited | Yes | Yes |
| DSP Instructions | No | No | No | No | Yes |
| Floating Point Unit | No | No | No | No (M4F variant has FPU) | Optional |