emvedded linux develment of rasberry pi(confirgurationa nd all)

Embedded Linux Development on Raspberry Pi (Configuration & Setup)

1. What is Embedded Linux?

Embedded Linux is a customized Linux operating system running on embedded hardware such as a Raspberry Pi. It allows developers to build applications, device drivers, and complete embedded systems.

2. Raspberry Pi Hardware Overview

5

A Raspberry Pi consists of:

- SoC (System on Chip) – CPU, GPU, memory controller

- RAM

- GPIO pins

- USB ports

- Ethernet/Wi-Fi

- SD card storage

- HDMI display interface

Common models:

| Model | CPU |
| --- | --- |
| Raspberry Pi 3 Model B | ARM Cortex-A53 |
| Raspberry Pi 4 Model B | ARM Cortex-A72 |
| Raspberry Pi 5 | ARM Cortex-A76 |

3. Development Environment Setup

Method 1: Native Development

Compile programs directly on Raspberry Pi.

sudo apt update
sudo apt install build-essential git

Compiler:

gcc hello.c -o hello
./hello

Method 2: Cross Compilation

Compile on PC and run on Pi.

Example toolchain:

arm-linux-gnueabihf-gcc

Compile:

arm-linux-gnueabihf-gcc hello.c -o hello

Transfer:

scp hello pi@192.168.1.10:/home/pi

Run:

./hello

4. Raspberry Pi OS Installation

Download Raspberry Pi OS

Use:

Raspberry Pi Imager

Steps:

- Insert SD card.

- Open Raspberry Pi Imager.

- Select OS.

- Select Storage.

- Write image.

5. First Boot Configuration

After boot:

sudo raspi-config

Configure:

- Hostname

- Password

- SSH

- Wi-Fi

- Locale

- Timezone

- Camera

- SPI

- I2C

- Serial Interface

6. Network Configuration

Check IP:

ip addr

Ping test:

ping google.com

Enable SSH:

sudo systemctl enable ssh
sudo systemctl start ssh

Connect from PC:

ssh pi@<ip-address>

7. GPIO Development

Install GPIO library:

sudo apt install gpiod

GPIO commands:

gpioinfo

gpioget gpiochip0 17

gpioset gpiochip0 17=1

8. Device Tree

Device Tree describes hardware to Linux.

Location:

/boot/firmware/

Files:

*.dtb

Compile:

dtc -I dts -O dtb file.dts -o file.dtb

9. Kernel Configuration

Download kernel:

git clone --depth=1 https://github.com/raspberrypi/linux

Configure:

make menuconfig

Build:

make -j4

Important options:

- Device Drivers

- Networking

- Filesystems

- USB Support

- I2C

- SPI

- GPIO

10. Boot Process

6

Raspberry Pi Boot Sequence

Power ON
   ↓
Boot ROM
   ↓
GPU Firmware
   ↓
config.txt
   ↓
Kernel Image
   ↓
Device Tree
   ↓
Root File System
   ↓
init/systemd
   ↓
User Applications

11. Kernel Module Development

Simple Driver:

#include <linux/module.h>

static int __init hello_init(void)
{
    printk(KERN_INFO "Hello Driver\n");
    return 0;
}

static void __exit hello_exit(void)
{
    printk(KERN_INFO "Driver Removed\n");
}

module_init(hello_init);
module_exit(hello_exit);

MODULE_LICENSE("GPL");

Compile:

make

Insert:

sudo insmod hello.ko

Check:

dmesg | tail

Remove:

sudo rmmod hello

12. Embedded Linux Filesystem

Important directories:

/
├── bin
├── boot
├── dev
├── etc
├── home
├── lib
├── proc
├── root
├── sys
├── tmp
├── usr
└── var

Useful commands:

ls
pwd
mount
df -h
free -m
top
ps

13. Communication Interfaces

UART

sudo raspi-config

Enable Serial Port.

Device:

/dev/ttyAMA0

I2C

Enable:

sudo raspi-config

Install:

sudo apt install i2c-tools

Detect devices:

i2cdetect -y 1

SPI

Enable SPI:

sudo raspi-config

Check:

ls /dev/spidev*

14. Build Systems

Popular Embedded Linux Build Systems:

| Tool | Purpose |
| --- | --- |
| Buildroot | Lightweight Linux generation |
| Yocto Project | Industrial Linux development |
| OpenEmbedded | Base layer for Yocto |
| BusyBox | Small command set |

15. Typical Embedded Linux Development Flow

Hardware Selection
       ↓
Install Raspberry Pi OS
       ↓
Configure Network & SSH
       ↓
Cross Compiler Setup
       ↓
Application Development
       ↓
GPIO/I2C/SPI/UART Programming
       ↓
Kernel Driver Development
       ↓
Build Custom Kernel
       ↓
Build Root Filesystem
       ↓
Deploy & Test

Viva/Interview Questions

- What is Embedded Linux?

- What is a Device Tree?

- Difference between Bootloader and Kernel?

- What is Cross Compilation?

- What is GPIO?

- What is Device Driver?

- What is Buildroot?

- What is Yocto?

- Explain Raspberry Pi boot process.

- Difference between user space and kernel space?

These topics cover the complete Embedded Linux development workflow on Raspberry Pi typically expected in embedded systems labs, interviews, and project work.

Locate kernel image and find is size

To locate the Linux kernel image on a Raspberry Pi and find its size:

1. Locate the Kernel Image

Newer Raspberry Pi OS versions store kernel files in:

ls -lh /boot/firmware/kernel*

Older versions use:

ls -lh /boot/kernel*

Example output:

-rwxr-xr-x 1 root root 8.2M Jun 20 10:00 /boot/firmware/kernel8.img

Common kernel images:

| File | Description |
| --- | --- |
| kernel.img | ARMv6/ARMv7 kernel |
| kernel7.img | 32-bit ARMv7 kernel |
| kernel7l.img | ARMv7 with LPAE |
| kernel8.img | 64-bit ARMv8 kernel |

2. Find Kernel Image Size

Using ls:

ls -lh /boot/firmware/kernel8.img

Or using du:

du -h /boot/firmware/kernel8.img

Or exact size in bytes:

stat -c %s /boot/firmware/kernel8.img

Example:

8617984

which is approximately:

8617984 / 1024 / 1024 ≈ 8.22 MB

3. Find Currently Running Kernel Version

uname -r

Example:

6.12.34+rpt-rpi-v8

4. Verify Which Kernel Image is Being Used

Check /boot/firmware/config.txt:

grep "^kernel" /boot/firmware/config.txt

Example:

kernel=kernel8.img

If no kernel= line exists, the Raspberry Pi firmware automatically chooses the appropriate kernel image for the hardware.

For a lab/demo, a single command that shows both name and size is:

ls -lh /boot/firmware/kernel*

This lists all kernel images along with their sizes.

not rasberry pui, n eneral

On a general Linux system (not Raspberry Pi), you can locate the currently running kernel image and find its size as follows.

1. Check the Running Kernel Version

uname -r

Example:

6.8.0-60-generic

2. Locate the Kernel Image

Most distributions store kernel images in /boot.

ls /boot/vmlinuz*

Example:

/boot/vmlinuz-6.8.0-60-generic

To locate the kernel corresponding to the running version:

ls -l /boot/vmlinuz-$(uname -r)

3. Find the Kernel Image Size

Human-readable size:

ls -lh /boot/vmlinuz-$(uname -r)

Example:

-rw-r--r-- 1 root root 15M Jun 10 12:30 /boot/vmlinuz-6.8.0-60-generic

Exact size in bytes:

stat -c %s /boot/vmlinuz-$(uname -r)

or

du -h /boot/vmlinuz-$(uname -r)

4. Find All Installed Kernel Images and Their Sizes

ls -lh /boot/vmlinuz*

Example output:

-rw-r--r-- 1 root root 14M vmlinuz-5.15.0
-rw-r--r-- 1 root root 15M vmlinuz-6.8.0

5. Verify Kernel Image Used During Boot

On GRUB-based systems:

grep linux /boot/grub/grub.cfg

or

grep menuentry /boot/grub/grub.cfg

This shows which vmlinuz files are available for boot.

One-line Answer (often expected in labs)

ls -lh /boot/vmlinuz-$(uname -r)

This command displays the currently running kernel image and its size.